[image: image1.png]| www.panini.com





My Vision X – Win XP & Win 2000 - Manual Installation
Updated July 18, 2007

These instructions explain what files are included in the My Vision X (MVX) software driver package and what files are needed to support the MVX API.  The instructions also explain step by step how to manually install and uninstall these files on a system running the Windows XP 32 bit or Windows 2000 Pro operating system. 

To manually install any MVX API version 2.x.x.x just copy the desired version of the MVXAPI.dll file as directed in the instructions below.  The MVXAPI.DLL is the application program interface to the MVX.  All 2.x.x.x versions of the MVXAPI.DLL use the same "driver package" files and the same Wd_utils.dll, Baroc.dll and AXBAR32.dll files.  The desired MVXAPI.dll file can be obtained from a system after running the appropriate Panini supplied InstallShield installation. 

To install the MVX 2.x.x.x driver and API files on a system without previous MVX software drivers installed
A. Preinstall the MVX software before connecting the MVX to the system:

1.  Copy the MVX WHQL driver and setup files to a single "driver package" directory on the system.  This includes the 5 driver and support files signed by Microsoft and the cat file which contains the WHQL signature.

These files are:

MVX.inf

(for installing the MVX USB Device IDs)

Windrvr6.inf

(for installing the Windriver6 service and device driver)

Windrvr6.sys

(the USB driver, version 7.0.1.0 for Win XP & Win 2000)

MvxCoinst.dll

(supports install process)

Wdreg.exe

(used to install/uninstall the inf files)

Myvisionx.cat

(Microsoft signature file for the driver package files)

This directory and these files must remain on the system.  Files in this directory are used by Plug and Play when the MVX USB is first plugged in.  The files are also used when this driver is uninstalled.

2. Copy the desired MVXAPI.dll version to the windows\system32 dir.

3. Copy Wd_utils.dll (supports MVXAPI.dll) to the windows\system32 dir.

4. Copy the Baroc.dll (supports OCR fonts OCRA, OCRB, E13B) to the Windows\system32 dir if the MVX OCR function will be used. 

5. Copy the AXBAR32.dll file to the Windows\system32 dir if the MVX 1D barcode function will be used. 

6. Run the Copy_inf.exe or call SetupCopyOEMinf() for the MVX.inf file.  Be sure you put the full path to the MVX.inf file into the call or command line if using Copy_inf.exe.  Copy_Inf makes the call to SetupCopyOEMinf().

SetupCopyOEMInf() copies the MVX.inf file into the Windows\inf path renaming it to oemXX.inf and also saves the path to the other Driver Package files so that Plug and Play can find these files when it is time to complete installation of the driver files.

The name of the oemXX.inf file created should be saved to file for use by the MVXUninstall.bat file should it be desired to delete the driver.  This function should be added to the Copy_Inf utility.

See the MVXSetup.bat file for an example of using Copy_Inf.

B. After installing the driver and API software, then plug in the MVX USB:

When the MVX USB is first plugged in, the following events take place:

Plug and Play sees the MVX USB ID (Vid_121f&Pid_0001) for the "without firmware" state.  Plug and Play then finds the inf file that supports this device ID and uses instructions in the inf file to install the driver to support the MVX in this state.

Several small "Found New Hardware" popups will appear briefly, then close.  But no prompts requiring user input should appear.

Device Manager should show the "MyVisionX without firmware" driver without a problem under USB Controllers.

After plugging in the USB cable, the user should wait 5-10 seconds for the plug and play logic to complete installing the MVX driver in the "MyVisionX without firmware" state before running a MVX application for the first time.
C. After the MVX USB and power are plugged in and Plug and Play has installed the MVX in the "without firmware" state, then run an MVX application:
When an MVX application is first run and calls MVX API Startup, the following events take place:

When the application calls MVX StartUp, the dll copies the firmware to the MVX, then forces the MVX to renumerate (change USB ID).  This causes Plug and Play to install the driver to support the MVX in the "with firmware" state (USB ID = Vid_121f&Pid_0002).

Several small popups should appear briefly, but then close without any user actions.

The MVX will initialize, and the Sorter Connected message will be received by the application.

Device Manager should show the "MyVisionX with firmware" driver without a problem under USB Controllers.

Note that any time an MVX is powered down or its USB is disconnected, it reverts to the without firmware state.

Uninstalling the version 2.x.x.x driver and support files:

See the MVXUninstall.bat file supplied for an example of using the Wdreg utility to uninstall the driver, inf file and the links in the registry that link the USB IDs to the driver.

The path in the MVXUninstall.bat file should be edited before running this batch file so it contains the correct full path and filename of the oemXX.inf file created when SetupCopyOEMInf was called.

It is important that the old registry USB ID to driver links be uninstalled before trying to install a different version of the MVX USB drivers such as used for version 1.63.1.2 or prior.

When these links are installed, they can be found in registry keys:

  HKLM\System\CurrentControlSet\Enum\USB\Vid_121f&Pid_0001

  HKLM\System\CurrentControlSet\Enum\USB\Vid_121f&Pid_0002

After running the MVXUninstall.bat file, edit then run the WinDrvr6Uninstall.bat file to remove the registry entries for the WinDrvr6 virtual device driver.  This batch file also removes the oemXX.bat file copy of the WinDrvr6.inf in the windows\Inf directory.
The path in the WinDrvr6Uninstall.bat file should be edited before running this batch file so it contains the correct full path and filename of the oemXX.inf file created when the winDrvr6
driver was installed.  This oemXX.inf file is an exact copy of WinDrvr6.inf.

To update the MVX API version on a system with previous drivers installed

To change between any two MVX API 2.x.x.x versions you can simply copy the desired MVXAPI.dll version into the windows\system32 directory.  Then unplug and reconnect the MVX USB cable or power the system down and restart.
To go from MVX API 1.63.1.2 or prior version to any MVX 2.x.x.x version, run the MVX_16312_Uninstall.bat file to first remove the old USB drivers and API dlls.  See the file “MVX v1.63.1.2 Manual Uninstaller.txt” for information about this bat file.  Then run the process to install the MVX 2.x.x.x drivers and API files as described above.

Further Information
Further information about manually installing device drivers can be found in MSDN article "Preinstalling Driver Packages" in the section "Guidelines for Writing a Device Installation Application".  This article is available at http://MSDN2.microsoft.com.

PAGE  
	[image: image2.wmf]

	MVX - WinXP Win2000 - Manual Install.doc
1




[image: image2.wmf]